Tavano Matteo											SM3800057

Cloud Basic Assignment – Performance Testing

Introduction:

This project focuses on the performance evaluation of cloud infrastructure, specifically assessing the deployment of Virtual Machines (VMs) using the Oracle VM VirtualBox hypervisor and containerization with Docker. The performance tests conducted in both environments aim to analyze key metrics, including CPU utilization, memory consumption, disk read/write operations, and network communication between nodes within a subnet.

The primary goal of this project is to conduct a comprehensive performance assessment of cloud-based infrastructures by deploying Virtual Machines (VMs) via the Oracle VM VirtualBox hypervisor and containerized environments using Docker. The evaluation aims to benchmark and compare key performance indicators. This analysis will provide insights into the computational overhead, resource efficiency, and overall scalability of both virtualization and containerization approaches in cloud computing environments.

Part I: Virtual Machines Performance Test

Setup:

To facilitate the performance evaluation, a virtualization environment was established using Oracle VM VirtualBox as the chosen hypervisor. A baseline virtual machine, referred to as Template, was created with a configuration of 1 vCPU, 1 GB of RAM, and a 25 GB virtual hard disk. The operating system selected for deployment was Ubuntu 24.04 LTS Server.

https://ubuntu.com/download/server.

To enable internal network communication between virtual machines, the Template was cloned into three distinct instances: Master, Node01, and Node02. Each of these virtual machines was scaled up 2 vCPUs, 2 GB of RAM, and maintained a fixed system configuration to ensure consistency in performance testing. This uniform allocation of resources across the VMs is a critical factor in obtaining reliable benchmarking results.

After cloning the three virtual machines, specific network configurations were modified within Oracle VM VirtualBox to facilitate connectivity. Each VM was configured with two network interfaces: the first interface (Scheda1) was set to NAT mode to enable external connectivity, while the second interface (Scheda2) was assigned to an Internal Network named clustervimnet. This configuration establishes a shared network environment, allowing the virtual machines to communicate using local IP addresses within the cluster.

Install Dependencies:

- sudo apt update

- sudo apt upgrade -y

- sudo apt install -y openssh-server net-tools gcc make build-essential gfortran openmpi-bin openmpi-common libopenmpi-dev

- sudo systemctl enable --now ssh

- sudo systemctl start ssh

To ensure effective communication within the cluster, the network configuration of each virtual machine was modified. This required editing the 50-cloud-init.yaml file located in the /etc/netplan/ directory. The configuration file was updated as follows:

- sudo nano /etc/netplan/50-cloud-init.yaml

Next in this report, it is set another possible configuration.

Interface enp0s8: Assigned a static IP address for each node within the clustervimnet internal network:

- Master: 192.168.0.1

- Node01: 192.168.0.2

- Node02: 192.168.0.3

Interface enp0s3: Configured to use DHCP (dhcp4: true) to enable NAT-based external connectivity.

The configuration changes were applied using the following command to ensure proper network functionality:

- sudo netplan apply

To validate the network configuration (details of which will be discussed later), connectivity between virtual machines can be tested using the ping command. For example, from the Master node, the following command can be executed:

- ping -c 4 192.168.0.2

If the setup is correctly configured, Node01 should receive and respond to the packets sent from the Master node, confirming successful communication within the internal network.

- Configure Master Node

Set the host name:

To ensure correct node identification, set the hostname of the master node:

- Sudo hostname set-hostname master

Edit the hosts file:

The /etc/hosts file needs to be edited to define static IP mappings for all nodes:

- Sudo nano /etc/hosts

Install and Configure Dnsmasq

To provide DHCP and DNS services for the cluster:

- sudo apt install -y dnsmasq

Edit the Dnsmasq configuration file:

- sudo nano /etc/dnsmasq.conf

Configure DNS Resolution:

Create a new resolv.conf file with the correct DNS settings:

- sudo nano /etc/resolv.conf

Enable External Access and Reboot:

To ensure external access is available:

- sudo ln -s /run/systemd/resolve/resolv.conf /etc/resolv.dnsmasq

- sudo systemctl restart dnsmasq systemd-resolved

- sudo systemctl enable dnsmasq

- sudo reboot

- Setup Distributed File System

Install NFS Server on Master Node:

- sudo apt install nfs-kernel-server

Create a shared directory accessible to all nodes:

- sudo mkdir /shared

- sudo chmod 777 /shared

Configure NFS Exports:

Edit NFS configuration file:

- sudo nano /etc/exports

Add the entry:

- /shared/ 192.168.0.0/255.255.255.0(rw,sync,no_root_squash,no_subtree_check)

Restart NFS Server:

- sudo systemctl enable nfs-kernel-server

- sudo systemctl restart nfs-kernel-server

- Configure and compute the nodes (Node01 – Node02)

Clone as mentioned before (use OracleVM setup menu)

Install Dependencies:

- sudo apt update

- sudo apt upgrade -y

- sudo apt install -y openssh-server net-tools libblas3 libblas-dev libopenmpi-dev openmpi-bin openmpi-common

Enable and start SSH:

- sudo systemctl enable –now ssh

- sudo systemctl start ssh

Configure network setting for nodes:

As for master node:

- sudo nano /etc/netplan/50-cloud-init.yaml

- sudo netplan apply

Set Hostnames for Node01

- sudo hostname set-hostname node01

Configure DNS for the Node:

Remove the existing resolv.conf symlink:

- sudo unlink /etc/resolv.conf

Create a new file:

- sudo nano /etc/resolv.conf

Add:

Edit the host file for Node01:

- sudo nano /etc/hosts

Repeat the steps for Node02;

- Enable Passwordless SSH Access

From the Master node generate a ssh key:

- ssh-keygen -t rsa

Copy to both nodes:

- ssh-copy-id user01@node01

- ssh-copy-id user01@node02

Now the master can access the nodes via SSH without requiring a password, facilitating distributed HPCC execution.

CPU Test HPC Challenge Benchmark:

In this section, the HPC Challenge (HPCC) Benchmark will be utilized to assess high-performance computing capabilities on Ubuntu-based systems. HPCC is a comprehensive benchmarking suite that evaluates processor performance, memory subsystem efficiency, and interconnect bandwidth. It includes several tests, such as HPL, STREAM, DGEMM, PTRANS, RandomAccess, and FFT, each targeting different aspects of system performance. http://icl.cs.utk.edu/hpcc/

Key optimizationin High-Performance Linpack (HPL) benchmark parameters include block size (NB), which depends on the CPU architecture, as well as grid dimensions (P and Q) that should match the number of MPI processes. The problem size (N) is another crucial factor in performance tuning.

Downloading HPCC and Preparing the Environment: (version hpcc-1.5.0)

The HPCC benchmark suite needs to be downloaded and extracted in a shared directory to ensure accessibility across multiple nodes. Using a shared folder allows all nodes within the cluster to access the benchmark without redundant copies. The shared directory is mounted as /shared on all nodes.

- cd /shared

- wget https://icl.utk.edu/projectsfiles/hpcc/download/hpcc-1.5.0.tar.gz

- tar xzf hpcc-1.5.0.tar.gz

- cd hpcc-1.5.0

This sequence downloads the HPCC version 1.5.0 archive, extracts its contents, and navigates into the extracted directory.

Configuring the Build System:

The compilation process of HPCC requires an appropriate makefile. A template configuration file (Make.Linux_PII_CBLAS) is selected as the basis due to its compatibility with CBLAS and x86 architecture. The following command copies the template configuration file to the appropriate location:

- cp hpl/setup/Make.Linux_PII_CBLAS hpl/Make.Linux

Editing Make.Linux for the Correct Library Paths

The Make.Linux file must be modified to correctly specify paths for compilation and linking. This step ensures that the HPCC build system correctly locates necessary dependencies.

- nano hpl/Make.Linux

Makefile variables:

- TOPdir: Set to /shared/hpcc-1.5.0, ensuring all relative paths within the makefile correctly reference the shared HPCC directory.

- MPdir: Set to /, as OpenMPI's system-wide installation in Ubuntu places relevant libraries and headers under /usr.

- MPinc (MPI Include Paths): Extended to include OpenMPI headers along with HPCC and HPL-specific directories, ensuring correct compilation.

- MPlib (MPI Library Path): Adjusted to -L/usr/lib/x86_64-linux-gnu -lmpi, linking against OpenMPI’s dynamically linked libraries instead of static ones.

- LAdir (Linear Algebra Directory): Modified to /usr/lib/x86_64-linux-gnu, which contains the system-installed BLAS and LAPACK implementations.

- LAlib (Linear Algebra Library): Changed to -lblas, linking directly to the system-provided BLAS library rather than a custom-compiled one.

- Fortran Compatibility (F2CDEFS): Defined as -DAdd_ -DF77_INTEGER=int -DStringSunStyle, ensuring proper name mangling and integer type consistency between Fortran and C.

- LINKER: Set to $(CC), ensuring a consistent compiler toolchain is used.

- RANLIB: Updated to ranlib to utilize standard library indexing.

Compiling HPCC:

Once the Make.Linux file is correctly configured, it is copied to the HPCC root directory to be used during the build process:

- cd /shared/hpcc-1.5.0

- cp hpl/Make.Linux Make.Linux

Call makefile:

- make arch=Linux

This command builds the HPCC executable, compiling the benchmark suite with the specified settings.

Configuring the Hostfile for MPI Execution

To enable multi-node execution, a hostfile is created that specifies the participating nodes. This file informs OpenMPI of the available compute nodes and their resource allocation.

- Sudo nano /shared/hpcc-1.5.0/hostfile

The following entries are added:

Each node is assigned one processing slot, allowing OpenMPI to distribute processes accordingly.

Mounting the Shared Folder on Compute Nodes

Before executing HPCC, the shared folder containing the benchmark must be accessible on all nodes. This is achieved by mounting the shared directory from the master node:

- Sudo mount master:/shared /shared

This ensures that the executable and required files are available across all participating nodes.

Launching the HPCC Benchmark

The HPCC benchmark is executed using OpenMPI with the following command:

- /usr/bin/mpirun -np 2 --display-map --map-by node --hostfile hostfile \ --mca btl tcp,self --mca btl_tcp_if_include enp0s8 -v ./hpcc > hpcc_summary.txt 2>&1

The execution involves several key configurations:

- Process Distribution: The -np 2 option specifies two parallel processes, which are distributed across nodes as dictated by --map-by node.

- Network Configuration: The --mca btl tcp,self flag enforces the use of the TCP transport layer for inter-node communication. The --mca btl_tcp_if_include enp0s8 option ensures communication occurs over the specified network interface (enp0s8).

- Process Mapping: The --display-map flag displays how processes are mapped across nodes, aiding in debugging and performance analysis.

- Hostfile Utilization: The --hostfile hostfile argument ensures OpenMPI distributes processes according to the predefined node allocation.

- Verbose Output: The -v flag enables detailed logging.

- Output Redirection: Standard output and error messages are captured in hpcc_output.txt for later inspection.

Upon completion, the output file can be analyzed to assess the performance of the benchmark, providing insights into computational efficiency and inter-node communication latency.

Comments on the result file (hpcc_summary.txt)

The performance benchmark results for the HPCC test suite on the virtualized cluster indicate modest computational capabilities with some variability across different metrics. The HPL benchmark shows a peak performance of around 0.0077 TFlops, which is relatively low, likely due to resource constraints imposed by the virtualization environment and limited CPU and memory per node. STREAM memory bandwidth results vary significantly, with some measurements reaching over 100 GB/s in smaller vector sizes, but overall performance remains inconsistent, suggesting memory access bottlenecks. DGEMM results hover around 3–4 GFlops, reflecting the limited floating-point processing power available. PTRANS network bandwidth is moderate, peaking at approximately 6 GB/s in optimal conditions, but generally remains below 1 GB/s, highlighting inter-node communication limitations. RandomAccess performance is notably low, with GUPs values well below expectations, indicating significant latency in random memory access operations. Overall, the benchmark results suggest that while the cluster can handle basic distributed computations, performance is constrained by virtualization overhead, network bandwidth, and memory access efficiency.

General System Test:

After completing the HPL benchmark, a general system performance test was conducted using the stress-ng and sysbench utilities. These tests were executed on the Master node and Node01 to assess system stability and resource utilization. The results, along with the command-line execution and parameter configurations, will be analyzed in the following sections.For this, stress-ng and sysbench has been downloaded.

- sudo apt update && sudo apt install stess-ng sysbench -y

The subsequent sections will present a summary of the results and include relevant screenshots captured during the testing process.

CPU TEST (SYSBENCH) on Master:

- sysbench cpu --cpu-max-prime=20000 --threads=2 run

CPU TEST (SYSBENCH) on Node:

- sysbench cpu --cpu-max-prime=20000 --threads=2 run

Comments on the results:

Metrics:

- Total number of events: The number of prime numbers calculated during the test.

- Latency (ms): Measures the response time of computations, with average, minimum, and maximum values reported. Lower values indicate better performance.

- CPU events per second: The number of completed operations per second, representing CPU efficiency.

- Execution time: The total duration of the test, including CPU and system overhead.

Comparison:

- Both nodes ran the test with two threads and a maximum prime number of 20,000, ensuring a direct comparison.

- The Master node completed slightly fewer CPU events per second compared to Node01, suggesting minor variations in performance.

- Latency values are similar, indicating comparable processing efficiency.

- The execution time is slightly different, possibly due to background processes or slight hardware variations.

Conclusions:

The results show that both nodes exhibit similar computational capabilities, with minor differences in CPU performance. These variations could result from differences in system load or scheduling during the test execution.

MEMORY TEST (SYSBENCH) on Master:

- sysbench memory –memory-total-size=2G run

MEMORY TEST (SYSBENCH) on Node:

- sysbench memory –memory-total-size=2G run

Comments on the results:

Metrics:

- Total operations performed: The number of memory operations executed during the test.

- Memory transfer rate (MB/sec): Measures how fast data is transferred in memory. Higher values indicate better memory performance.

- Latency (ms): The response time of memory operations, with average, minimum, and maximum values reported.

- Execution time: The total duration of the test.

Comparison:

- Both nodes performed the test with a total allocated memory size of 2GB, ensuring consistency.

- The total number of operations and transfer rate appear similar between the Master and Node01, indicating comparable memory bandwidth and efficiency.

- Latency values show only minor variations, suggesting similar memory response times.

- Execution time is nearly identical, confirming that both nodes exhibit comparable memory performance.

Conclusions:

The memory performance results show no significant differences between the Master and Node01, indicating consistent memory bandwidth and efficiency across both nodes. The minor variations observed could be attributed to system load fluctuations during testing.

CPU TEST (STRESS-NG) on Master:

- stress-ng –cpu 2 –cpu-method matrixprod –timeout 60s –metrics-brief

CPU TEST (STRESS-NG) on Node:

- stress-ng –cpu 2 –cpu-method matrixprod –timeout 60s –metrics-brief

Comments on the results:

Metrics:

- CPU operations (bogo ops): Measures the number of "bogus operations" performed as a synthetic workload, useful for relative comparisons.

- Real time, user time, and system time (secs): Real time is the total duration of the test, User time is the CPU time spent executing user-space processes and System time is the CPU time spent executing kernel operations.

- Bogo ops per second: A relative measure of computational throughput. Higher values indicate better performance.

Comparison:

- Both tests were run using two CPU threads and the matrixprod method for 60 seconds, ensuring a direct comparison.

- The Master node achieved a higher number of total CPU operations (327,474 vs. 133,892), indicating significantly better computational throughput.

- The bogo ops per second on the Master (6,212.25) is almost three times higher than on Node01 (2,202.64), suggesting a performance gap.

- User and system times on the Master are also notably higher, aligning with its increased workload processing efficiency.

Conclusions:

The Master node demonstrated substantially better CPU performance compared to Node01 under the same test conditions. The discrepancy could be attributed to differences in background processes, CPU scheduling, or potential hardware variations

MEMORY TEST (STRESS-NG) on Master:

- stress-ng –vm 1 –vm-bytes 1500M –timeout 60s –metrics-brief

MEMORY TEST (STRESS-NG) on Node:

- stress-ng –vm 1 –vm-bytes 1500M –timeout 60s –metrics-brief

Comments on the results:

Metrics:

The memory stress test was performed using the stress-ng command with the vm parameter set to 1, meaning a single worker allocated and deallocated memory. The following metrics were measured:

- bogo ops: number of operations performed by the test.

- real time (s): actual elapsed time to complete the test.

- user time (s): CPU time spent in user mode.

- sys time (s): CPU time spent in kernel mode.

- bogo ops/s: operations performed per second.

Comparison:

- The Master executed 510,000 bogo ops, with a real-time duration of 37.53s, user time of 22.07s, and kernel time of 8444.08s, achieving a rate of 8556.63 bogo ops/s.

- The Node executed 1,787,539 bogo ops, with a real-time duration of 19.94s, user time of 49.80s, and kernel time of 23,663.03s, achieving a rate of 23,703.60 bogo ops/s.

Conclusions:

The Node performed significantly more operations than the Master, with over three times the number of bogo ops and a much higher execution speed. This suggests that the Node has better memory performance

Even if the Master and Node are configured identically, differences in performance can arise due to several factors. The Master might be running additional background processes or services that consume CPU and memory resources, leading to lower efficiency. Another factor could be CPU scheduling and NUMA effects, where differences in CPU-to-memory access paths influence performance. The Node may have a more optimal memory allocation due to the way the kernel assigns resources based on workload conditions. Additionally, memory management and caching behavior can vary, even with identical setups. Differences in page allocation, swap usage, and caching efficiency may result in the Node utilizing memory more effectively, reducing latency and increasing throughput.

Disk I/O Test:

This section of the report focuses on the Disk I/O performance test on the local file system. Disk I/O (Input/Output) testing measures the system's ability to read and write data to storage devices, evaluating factors such as speed, efficiency, and overall performance. For this purpose, the IOzone package on Ubuntu will be used. The test will be conducted on both the Master and Node01 to assess and compare their disk performance.

The package has been retrieved from:

- sudo apt install iozone3 -y

IOZONE TEST MASTER:

- iozone -a -g 1G

The screenshot shows the results of an IOzone test. The command above runs an automatic mode test (-a) with a maximum file size of 1GB (-g 1G). The results table displays various read and write performance metrics in (KB/s) for different file sizes and record sizes.

Key parameters include sequential and random read/write speeds, re-read/re-write performance, and various access patterns (stride, backward, record, etc.). The numbers in the table indicate throughput for each operation, where higher values generally mean better performance.

From the results, larger record sizes tend to yield higher throughput, which is expected as larger writes reduce the overhead of multiple small operations. These results help in evaluating disk performance, optimizing storage configurations, and identifying bottlenecks in file system operations.

IOZONE TEST NODE:

- iozone -a -g 1G

The IOzone test results shown in the screenshot were generated using the command iozone -a -g 1G, which runs in automatic mode while testing various file and record sizes with a maximum file size of 1GB.

The results provide throughput values in KB/s for different read and write operations, including sequential access, random access, backward reads, and strided reads. As expected, performance improves with larger record sizes since fewer operations are required per file. Sequential read and write speeds are significantly higher than random access speeds, highlighting the inefficiencies of non-contiguous disk access. The results also show that record rewrite performance is relatively strong, indicating efficient handling of repeated write operations. However, backward reads and strided accesses show lower throughput, suggesting that these access patterns may not be optimized for peak performance.

For a Node-based system, these results are important in assessing file system efficiency, optimizing storage configurations, and understanding caching behavior in high I/O workloads. In scenarios involving databases or large-scale applications, evaluating these performance metrics can help identify potential bottlenecks.

Network Test:

After completing the I/O performance tests, connectivity tests between nodes and the master was conducted to evaluate network performance. For this purpose,  tools such as HyPerf3, Netcat, and Ping was utilized to assess packet reception over TCP/IP, measure throughput, and record latency. The primary objective of this analysis was to ensure the accurate transmission and reception of data while obtaining key network performance metrics. The results are presented in detail, covering communication from Node to Master, Master to Node, and between the two nodes within our ClusterVimNet environment.

Ping Tests:

In this section, we conduct a ping test to evaluate network connectivity and latency between nodes within our environment. The test is performed across three distinct communication paths: from the master node to a worker node, from a worker node to the master, and between Node01 and Node02. Each test involves sending ICMP packets, each containing 64 bytes of data, to measure response times and packet reception. The transmitted packets are sequentially numbered within an ICMP sequence, and the results are analyzed based on minimum, average, and maximum round-trip times (RTT). Additionally, we assess the total transmission time and the percentage of successfully received packets. Screenshots of each test scenario are provided to verify the actual reception of packets. The following command has been executed:

- ping -c 10 192.168.0.x

ping ->  The command-line utility used to test network connectivity between a source and a destination by sending ICMP echo request packets.

-c  10 ->  This option specifies the number of packets to send. In this case, 10 packets will be transmitted before the command terminates.

192.168.0.x → This is the target IP address being tested. The placeholder x represents the specific last octet of the IP address for the destination node.

Master -> Node

Node -> Master

Node01 -> Node02

Iperf3 Network Bandwidth Test:

We will now utilize HyPerf3 to evaluate network throughput between the nodes within the clustervimnet environment. This process requires the installation of the necessary dependencies, including the HyPerf3 package, on an Ubuntu system. Once the tests are conducted, the results will be analyzed based on transmission time and bandwidth measurements, allowing for a detailed assessment of network performance.

Installing packages:

- sudo apt update && sudo apt install -y iperf3

Master -> Node

The throughput test was conducted using iPerf3 to evaluate network performance between the nodes. Initially, the receiving node was set to listening mode by running the command

- iperf3 -s

allowing it to accept incoming connections. The master node then initiated the test by transmitting data to the receiving node using

- iperf3 -c 192.168.0.2.

The results indicate a fluctuating bandwidth throughout the test, with variations in data transfer rates across different time intervals. The highest observed throughput occurred in the first second, reaching 701 Mbits/sec, while subsequent measurements demonstrated variations, with some intervals dropping as low as 200 Mbits/sec. The overall average bitrate over the 10-second duration was 394 Mbits/sec on the sender side and 391 Mbits/sec on the receiver side. The test also recorded 416 retransmissions, which may suggest network congestion or suboptimal performance conditions.

Node -> Master

Then, establishing a connection from Node to the Master node. The receiving node was initialized in listening mode using iperf3 -s, while Node acted as the client, transmitting data via iperf3 -c 192.168.0.1.

The results indicate an initial peak bitrate of 567 Mbits/sec, followed by fluctuations throughout the test duration. The average throughput recorded over the entire session was 411 Mbits/sec on the sender side and 409 Mbits/sec on the receiver side. Despite relatively stable bandwidth performance, the transmission recorded 2,163 retransmissions, suggesting potential congestion or packet loss affecting data integrity.

Node01 -> Node02

A throughput test was conducted using iPerf3, establishing a connection from Node01 to Node02. The receiving node was set to listening mode with iperf3 -s, while Node02 initiated the test as the client using iperf3 -c 192.168.0.3.

The results show an initial bitrate of 282 Mbits/sec, followed by variations across different time intervals, with throughput reaching a peak of 560 Mbits/sec in the final second. The average transmission rate over the 10-second test was 341 Mbits/sec on the sender side and 337 Mbits/sec on the receiver side. A total of 456 retransmissions were recorded, suggesting some degree of network congestion or packet loss during the test.

These performance metrics provide insight into the network stability and data transfer efficiency between Node01 and Node02, indicating relatively consistent throughput with minor fluctuations.

Netcat throughput test:

Finally, network throughput was evaluated using Netcat as the testing tool. Consistent with the previous assessments, three key test scenarios were examined, and the results were analyzed to assess data transmission performance and network efficiency.

To perform this test, the following command has been executed: (sender)

- time dd if=/dev/zero/ bs=1M count = 100 | nc 192.168.0.x 5000

receiver, should listen to the communication using:

- nc -l -p 5000 > /dev/null

Breakdown of Command Parameters:

- time → Measures the execution time of the command, providing insights into transmission duration.

- dd if=/dev/zero → Uses the dd command to generate a stream of data from /dev/zero, which produces a continuous flow of null bytes.

- bs=1M → Sets the block size to 1 megabyte, meaning data is processed in 1MB chunks.

- count=100 → Specifies that 100 blocks (each of 1MB) will be transmitted, resulting in a total data transfer of 100MB.

- | (pipe) → Directs the output of dd to the next command in the pipeline.

- nc 192.168.0.x 5000 → Uses Netcat (nc) to send the data to the specified IP address (192.168.0.x) on port 5000, where a receiving Netcat instance is expected to be listening.

This command measures the time required to transfer 100MB of data between nodes, allowing for an estimation of network throughput.

Master -> Node

Node -> Master

Node01 -> Node02

Since all three components are configured identically, the variations in transfer speeds likely stem from factors beyond basic configuration differences. The asymmetry between Master → Node (29.4 MB/s) and Node → Master (43.7 MB/s) suggests potential discrepancies in network routing, background load, or hardware performance differences. The Node01 → Node02 transfer (35.4 MB/s) falls between the two, indicating that link-specific conditions, transient congestion, or even kernel-level optimizations might be influencing throughput despite uniform configurations.

Part II: Container Performance Test

This section of the report examines the performance tests conducted on a set of containers. As with the virtual machines, the containers are initialized through a predefined configuration file, ensuring a consistent setup. An internal network is established to facilitate communication between them, while their hardware resources are deliberately constrained to assess performance under limited conditions.

Setup:

The initial phase involves detailing the container initialization process and specifying the required software components. Docker Desktop is utilized as the primary platform to enable an efficient containerization environment. ( https://www.docker.com/products/docker-desktop/ )

To define and configure a set of containers, both a docker-compose.yaml file and a Dockerfile are employed. The docker-compose.yaml file orchestrates the creation of three distinct containers—hpcc_master, hpcc_node01, and hpcc_node02—designed for performance testing. These containers are interconnected through an internal network, designated as my_custom_network, which operates within the 192.168.1.0/24 subnet. Each container is assigned a unique alias (Node01, Node02, and Master) and a corresponding IP address within this subnet.

In alignment with the predefined constraints, each container is allocated limited hardware resources, specifically 2 GB of memory and 2 CPU cores. The containers are instantiated using Ubuntu 24.04, mirroring the virtual machine configurations.

Docker-compose.yaml

The Dockerfile serves as a blueprint for the automated provisioning of essential libraries and tools required for performance testing within the containers. Its primary function is to define a structured sequence of commands that are executed during the container build process, ensuring the installation and configuration of the necessary dependencies.

In this context, the Dockerfile facilitates the pre-installation of benchmarking utilities such as Netcat, IOzone, and Sysbench, which are essential for assessing network performance, disk I/O, and system resource utilization, respectively. By specifying the installation of these tools within the container build process, the Dockerfile guarantees a consistent and reproducible environment for performance evaluations. Furthermore, it ensures that all dependencies are correctly set up before container deployment, eliminating the need for manual configuration post-deployment and enhancing the reliability of the testing framework.

Dockerfile

To enable SSH access, the configuration creates the necessary directories, generates an SSH key pair, and sets up passwordless authentication by copying the public key to authorized_keys. It also modifies the SSH client configuration to disable strict host key checking and preemptively creates an empty known_hosts file to prevent potential authentication prompts.

A shared directory is created at /shared to facilitate data exchange between containers. The /etc/hpcc directory is also created, and the HPL.dat file is copied into it. Additionally, a hostfile defining the compute nodes, node01 and node02, each assigned one slot, is generated to support MPI-based execution.

The container exposes port 22 to allow SSH connections. The environment variables OMPI_ALLOW_RUN_AS_ROOT and OMPI_ALLOW_RUN_AS_ROOT_CONFIRM are set to permit running MPI processes as the root user. Finally, the container launches the OpenSSH daemon (sshd) in the foreground as its primary process.

Usage

Build Docker Image

The workflow begins with the creation of a Docker image using the docker build command. This command constructs an image based on the instructions specified in the Dockerfile, which includes the base operating system, application dependencies, and necessary configurations. The -t flag is used to tag the image with a recognizable name, such as ubuntu-performance, facilitating easier reference in subsequent commands.

- docker-build -t hpcc_image .

Run containers

Once the image is successfully built, the next step involves deploying the containerized environment using docker-compose up -d. Docker Compose allows for the orchestration of multi-container applications by defining services, networks, and volumes within a docker-compose.yml file. The -d flag ensures that the containers run in detached mode, enabling the user to continue other operations while the services remain active in the background. This approach streamlines the management of complex environments by automating the startup sequence and dependency resolution.

- docker-compose up -d

Check running containers

After deployment, verifying the status of running containers is crucial. The docker ps command lists all active containers, displaying essential details such as container IDs, assigned ports, and running statuses. This information aids in monitoring the deployment and diagnosing potential issues related to network configurations or application behavior.

- docker ps

Interact with a running container

To interact with a running container, the docker exec -it hpcc_master bash command is employed. This command spawns an interactive terminal session within the specified container, allowing direct access to its filesystem and execution of administrative commands. The -it flags ensure that the session remains interactive, providing real-time input and output, which is particularly useful for debugging and manual configuration adjustments.

- docker exec -it hpcc_master bash

Terminate and remove container

Once the application lifecycle is complete or the environment requires a reset, the docker-compose down command is used to stop and remove the containers, networks, and volumes defined in the docker-compose.yml file. This command ensures that no residual processes or dependencies remain active, preventing resource leakage and maintaining a clean system state.

- docker-compose down

Remove a defined network

Finally, the docker network rm my_custom_network command is executed to remove a custom-defined network, if applicable. Docker automatically creates networks to facilitate inter-container communication, but in scenarios where a specific network was manually defined and is no longer required, this command helps in reclaiming system resources and avoiding network conflicts in future deployments.

- docker network rm my_custom_network

This sequence of commands exemplifies a structured approach to container management, from image creation to cleanup, ensuring a controlled and efficient environment for deploying containerized applications.

CPU Test HPC Challenge Benchmark:

Once the containerized system has been properly set up, performance testing must be conducted to ensure that the system is functioning correctly and to collect critical performance metrics. One of the key tools used for this evaluation is the HPC Challenge Benchmark (HPCC), a comprehensive benchmarking suite designed to assess the computational capabilities of high-performance computing (HPC) systems. HPCC operates at the system level, providing insights into computational performance, memory bandwidth, and network communication efficiency, similar to the benchmarking procedures previously conducted for virtual machines (VMs).

To facilitate this testing process, the HPCC benchmark suite has been integrated into the containerized environment through the Dockerfile. The necessary dependencies, including required libraries and configuration files, have been pre-installed during the container build process. This ensures that once the container is deployed, the benchmarking tests can be executed without additional setup, allowing for a streamlined evaluation of system performance.

By running HPCC within the containerized infrastructure, we can analyze key performance indicators such as floating-point operations per second (FLOPS), memory throughput, and inter-process communication efficiency. These metrics provide valuable insights into the computational efficiency and scalability of the containerized system, enabling comparisons with traditional VM-based setups and identifying potential optimizations for improved performance.

HPL.dat

The HPL.dat file was configured according to the guidelines provided in "Tweak HPL Parameters". The configuration parameters include:

- Number of nodes: 3

- Cores per node: 2

- Total memory per node: 2048 MB

- Block size: Specified based on tuning recommendations (192)

These settings were used as input to optimize High-Performance Linpack (HPL) execution.

Run test

- mpirun --allow-run-as-root -np 2  --display-map --map-by node \

--hostfile /etc/hpcc/hostfile \

--mca btl tcp,self \

--mca btl_tcp_if_include eth0 \

--wdir /etc/hpcc \

--output-filename /etc/hpcc/hpcc_summary.txt \

/usr/bin/hpcc

Once inside the master container, the specified MPI-based HPCC benchmark test was executed to evaluate computational performance across multiple nodes. The command utilizes mpirun to distribute workloads across two processes, mapping them by node to ensure execution on separate hosts. The --hostfile /etc/hpcc/hostfile option defines the participating compute nodes, while --mca btl tcp,self and --mca btl_tcp_if_include eth0 configure OpenMPI to use TCP-based communication over the specified network interface. The working directory is set to /etc/hpcc, ensuring that input files such as HPL.dat and configuration settings are correctly referenced. The execution results are redirected to hpcc_summary.txt, preserving benchmark outputs for analysis. This test measures floating-point performance, memory bandwidth, and communication latency, providing insights into cluster efficiency under high-performance workloads.

Results:

The HPCC benchmark results provide key performance metrics that assess computational efficiency, memory bandwidth, and communication latency within the containerized environment. High GFLOPS values in the HPL benchmark indicate effective floating-point computation, while STREAM benchmarks evaluate memory throughput, reflecting data movement efficiency. Low latency and high bandwidth in PTRANS and MPI tests highlight optimized inter-container communication, a crucial factor for distributed workloads. The results underscore how containerization introduces minimal overhead, ensuring near-native performance by leveraging resource isolation and efficient networking, making it a viable solution for scalable, high-performance computing workloads.

General System Test:

After completing the HPL benchmark, a general system performance test was conducted using the stress-ng and sysbench utilities. These tests were executed on the Container1 to assess system stability and resource utilization. The results, along with the command-line execution and parameter configurations, will be analyzed in the following sections.  First, we need to execute the container to perform the test.

- docker exec -it hpcc_master

CPU TEST (SYSBENCH):

Sysbench evaluates CPU performance by calculating prime numbers up to 20,000 using two parallel threads. The key metrics include an event processing rate of 3,089.76 events per second and a total execution time of approximately 10 seconds. The latency distribution shows minimal variation, with an average latency of 0.65 ms and a maximum of 1.51 ms. These results indicate that the system maintains consistent computational efficiency under multithreaded workloads, demonstrating stable CPU performance with negligible thread fairness deviation.

- sysbench cpu --cpu-max-prime=20000 --threads=2 run

MEMORY TEST (SYSBENCH):

The second test measures memory performance using Sysbench by allocating and processing a total of 2GB of memory. The system achieves a transfer rate of 2931.53 MiB/sec, highlighting efficient memory bandwidth utilization. The latency is exceptionally low, with an average of 0.221 microseconds, indicating near-instantaneous memory access. The lack of significant latency spikes confirms stable memory performance, ensuring that memory-bound operations are handled with minimal overhead, crucial for high-performance computing applications.

- sysbench memory –memory-total-size=2G run

CPU TEST (STRESS-NG):

For CPU stress testing, Stress-NG employs the matrix product method with two CPU stressors for a duration of 60 seconds. The system processes approximately 71,851 bogo operations per second, with a total real-time execution of 60 seconds and accumulated user+system time of 1,197.58 seconds. These results demonstrate the system's ability to sustain heavy CPU workloads, effectively utilizing computational resources while maintaining predictable execution times. The relatively high system time compared to user time suggests an intensive computational burden, testing processor efficiency under synthetic loads.

- stress-ng –cpu 2 –cpu-method matrixprod –timeout 60s –metrics-brief

MEMORY TEST (STRESS-NG):

The final test evaluates memory stress using Stress-NG with a single virtual memory stressor, allocating and manipulating 1.5GB of memory over 60 seconds. The system achieves 6,799,569 bogo operations with a user+system time accumulation of 113,499.49 seconds. The low system time relative to user time suggests efficient memory access patterns, and the high bogo operation count reflects sustained memory throughput. These results indicate that the system effectively manages large-scale memory operations under stress, maintaining stable memory performance without significant slowdowns.

- stress-ng –vm 1 –vm-bytes 1500M –timeout 60s –metrics-brief

Disk I/O Test:

This section of the report focuses on the Disk I/O performance test on the local file system. Disk I/O (Input/Output) testing measures the system's ability to read and write data to storage devices, evaluating factors such as speed, efficiency, and overall performance. For this purpose, the IOzone package on Ubuntu will be used. The test will be conducted on Container1  to assess  disk performance.

- docker exec -it container1

The Iozone test executed with the -a -g 1G parameters performs a comprehensive file I/O benchmark within the Docker container, analyzing read, write, and access patterns across different file sizes up to 1GB. The test operates in auto mode, systematically measuring sequential and random access latencies, rewrite efficiency, and various read/write strategies. The dataset surpasses typical processor cache sizes, ensuring that the results reflect genuine disk or storage performance rather than CPU caching effects.

The results show that sequential read and write operations achieve high throughput, with read speeds exceeding 10,821,524 kB/s for larger block sizes and sequential writes maintaining substantial performance. Random read and write operations exhibit lower bandwidth, as expected, given the increased seek times and fragmentation overhead. The random read performance stabilizes around 7,904,539 kB/s, while random write operations register slightly lower speeds, influenced by storage medium characteristics and filesystem journaling overhead. Strided access patterns indicate performance consistency, with stride read speeds averaging 5,389,653 kB/s, suggesting efficient large-block handling.

These findings provide insights into the containerized environment’s I/O capabilities, revealing that the underlying storage subsystem performs efficiently under controlled workloads. The performance profile suggests that the container's filesystem configuration and storage backend introduce minimal bottlenecks, supporting demanding data-intensive applications.

Network Test:

After completing the I/O performance tests, connectivity tests between containers was conducted to evaluate network performance. For this purpose, I utilized tools such as HyPerf3, Netcat, and Ping to assess packet reception over TCP/IP, measure throughput, and record latency. The primary objective of this analysis was to ensure the accurate transmission and reception of data while obtaining key network performance metrics. The results are presented in detail, covering communication between the two containers within our my_custom_network environment.

Ping Test

The ping test evaluates basic network connectivity and latency between containers. The command ping -c 10 192.168.1.3 sends ten ICMP echo requests from the hpcc_master (192.168.1.2) to the target container (192.168.1.3). The results indicate stable connectivity, with minimal packet loss and consistent round-trip times. The reported latency values are crucial for assessing response delays, which in this test appear to be low, suggesting an efficient internal network configuration without significant congestion or interference.

- ping -c 10 192.168.1.3

Iperf3 Network Bandwidth Test:

The iperf3 bandwidth test measures network throughput between two containers, where one acts as a server (iperf3 -s) and listens on port 5201, while the other operates as a client (iperf3 -c 192.168.1.3 -p 5201), generating traffic towards the server. This test quantifies the available bandwidth, providing insights into network performance under load. The results display high throughput, confirming that the internal container network is capable of handling substantial data transfer rates. Variability in results could be attributed to network stack processing, available system resources, or overlay network configurations in Docker.

- iperf3 -s 5201

- iperf3 -c 192.168.1.3 5201

Netcat throughput test:

The netcat throughput test evaluates raw data transfer performance between two containers using nc. The server container listens on port 5000 (nc -l -p 5000 > /dev/null), while the client container transmits data using dd (dd if=/dev/zero bs=1M count=100 | nc 192.168.1.3 5000). This method streams 100MB of zeroed data to the server, measuring the transfer speed in real-time. The reported speed of approximately 7.96 MB/s suggests stable transfer rates, but performance may vary based on network stack optimizations, buffer sizes, and resource contention. The netcat test is effective for assessing point-to-point data transmission efficiency in a containerized environment.

- nc -l -p 5000 > /dev/null

- time dd if=/dev/zero bs=1M count=100 | nc 192.168.1.3 5000

Note: before performing network tests, ensure to enter both sender and receiver container, using docker exec command.

Conclusions

The benchmark tests conducted on the containerized system provide valuable insights into the performance characteristics of containers compared to traditional virtual machines (VMs). Each test evaluates a specific aspect of computational efficiency, including high-performance Linpack (HPL) for floating-point operations, CPU stress testing using Sysbench and Stress-NG, disk I/O assessment via IOzone, and network performance analysis with tools like ping, Netcat, and iPerf3. These tests highlight fundamental differences between containers and VMs in terms of performance overhead, resource efficiency, and practical deployment scenarios.

The HPL test results demonstrate that containerized environments deliver near-native performance for floating-point computations. Since containers share the host operating system's kernel, they introduce minimal overhead, allowing for more direct access to CPU resources. This enables high computational efficiency, making containers an attractive choice for workloads demanding high floating-point throughput. In contrast, VMs operate with a hypervisor layer that introduces additional latency and resource allocation inefficiencies, which can lead to slight performance degradation in HPL benchmarks. However, VMs provide better isolation and security, ensuring that workloads remain unaffected by system-wide kernel modifications or vulnerabilities.

CPU testing using Sysbench and Stress-NG further reinforces the efficiency of containers in utilizing processor resources. Containers, due to their lightweight architecture, execute CPU-bound tasks with minimal overhead, leveraging the host's computational power without significant performance loss. The absence of full-fledged virtualization layers results in lower latency and faster execution times for intensive workloads. In comparison, VMs experience additional CPU scheduling overhead imposed by the hypervisor, reducing overall performance. While modern hypervisors have been optimized to minimize these inefficiencies, containers still exhibit superior responsiveness and resource utilization, particularly in environments requiring rapid scaling and real-time processing capabilities.

Disk I/O performance, evaluated using IOzone, illustrates another key advantage of containerized systems. Containers benefit from direct access to the host filesystem or dedicated storage volumes with minimal abstraction, ensuring high read and write speeds. The reduced virtualization overhead allows for better disk throughput and lower I/O latency. Conversely, VMs rely on virtualized disk images, which introduce additional layers of abstraction between the application and physical storage. This can lead to performance bottlenecks, particularly when dealing with large-scale data operations. Although VMs offer better isolation and are often preferred in environments where data security is a primary concern, containers remain the preferred choice for high-performance, I/O-intensive applications due to their lower overhead and more efficient storage access.

Network performance testing with ping, Netcat, and iPerf3 provides insights into communication latency and bandwidth utilization within containerized environments compared to VMs. Containers exhibit lower network latency due to their lightweight architecture and the use of high-speed virtual networking mechanisms. Communication between containers on the same host is particularly efficient, as it bypasses the need for complex virtual network translations. On the other hand, VMs introduce additional network abstraction layers, often resulting in higher latency and reduced throughput. While modern hypervisor-based networking solutions have improved VM network performance, containers still maintain an edge in scenarios requiring low-latency, high-bandwidth communication, such as distributed computing and real-time data processing.

Despite the evident performance advantages of containers, VMs retain certain strengths that make them indispensable in specific use cases. One of the most significant benefits of VMs is their robust isolation and security model. Because VMs include a separate guest operating system for each instance, they provide strong isolation between workloads, preventing issues such as kernel-level vulnerabilities and cross-container security risks. This makes VMs ideal for environments that require stringent security controls, such as multi-tenant cloud deployments and regulatory-compliant infrastructures. Containers, although efficient, share the host kernel, making them more susceptible to security risks if not properly managed.

Another advantage of VMs is their ability to support a wide range of operating systems and configurations. Since VMs encapsulate an entire OS, they enable deployment of applications that require different kernel versions or system-level configurations that are not easily adaptable within a containerized setup. This flexibility is particularly useful for legacy applications or scenarios where software dependencies necessitate full system virtualization. In contrast, containers are limited to the host OS kernel, which can create compatibility challenges when dealing with diverse software environments.

On the other hand, containers excel in scalability, resource efficiency, and deployment speed. Due to their lightweight nature, containers can be instantiated or terminated rapidly, making them highly suitable for dynamic workloads that require elastic scaling. The ability to deploy multiple containers on a single host without the overhead of multiple OS instances significantly improves resource utilization.

The disadvantage of VMs in terms of resource consumption is another factor that makes containers preferable for high-performance computing environments. Since each VM requires a dedicated OS instance, memory and CPU resources are allocated inefficiently compared to containers, where only the application and necessary dependencies are packaged together. This results in higher RAM and disk usage per VM, reducing the overall system efficiency, especially in large-scale deployments. Containers mitigate this issue by sharing the host OS and utilizing resources more efficiently, leading to higher density and lower operational costs.

From a management perspective, containers also simplify application deployment and orchestration. With technologies like Docker and Kubernetes, containerized workloads can be easily orchestrated, scaled, and managed across distributed infrastructures. The declarative nature of container orchestration frameworks enables automated deployment, load balancing, and fault tolerance, which are more complex to achieve with traditional VM-based infrastructures. VMs, while benefiting from mature management tools such as VMware vSphere and OpenStack, require more overhead in provisioning, maintenance, and migration compared to containerized solutions.

Overall, the choice between containers and VMs depends on the specific workload requirements. Containers provide superior performance in CPU-bound tasks, disk I/O, and network communication due to their minimal overhead and direct access to system resources. They are particularly advantageous in scenarios requiring high scalability, fast deployment, and efficient resource utilization. However, VMs remain the preferred choice in environments where strong isolation, multi-OS support, and enhanced security are paramount. The results from the HPCC benchmarking tests reinforce the notion that while containers offer significant efficiency gains, they should be strategically deployed based on workload characteristics, security considerations, and infrastructure constraints.

References

- Cloud basic repository: https://github.com/Foundations-of-HPC/Cloud-basic-2024

- Ubuntu 24.04 server:  https://ubuntu.com/download/server.

- Oracle VM VirtualBox:  https://www.oracle.com/virtualization/technologies/vm/downloads/virtualbox-downloads.html

- Docker Desktop: https://www.docker.com/products/docker-desktop/

- HPC Challenge: https://hpcchallenge.org/hpcc/

- HPL tuning: https://www.netlib.org/benchmark/hpl/tuning.html

- Tweak HPL parameters: : https://www.advancedclustering.com/act_kb/tune-hpl-dat-file/

- Configure DNSMASQ: https://computingforgeeks.com/install-and-configure-dnsmasq-on-ubuntu/?expand_article=1

- Configure NFS Mount: https://www.howtoforge.com/how-to-install-nfs-server-and-client-on-ubuntu-22-04/

- SSH between containers: https://stackoverflow.com/questions/53984274/ssh-from-one-container-to-another-container

- sysbench: https://github.com/akopytov/sysbench

- stress-ng: https://manpages.ubuntu.com/manpages/bionic/man1/stress-ng.1.html

- IOzone: http://www.iozone.org/

- Iperf3: https://iperf.fr/